主标题：AC LED Neon Flex, Small Oval C-Bend

次标题： AC120V LED Neon flex,6W/3FT,08*16mm,160lm

- Product descriptions

- Uniform,absolutely homogeneous illumination,

- LM-80 approved LED of the premium quality with high brightness,

- High amount of copper in FPC ensure excellent heat dissipation,

- Resistant against water, UV radiation, chemicals and abrasion.

- Safe and simple connection system for easy on-site installation after cutting

- Estimated lifetime L70 at Ta＜45°C:＞50000h.

2. Product features

◆  Available in homogeneous white tones,start from 1800K

◆  a variety of light color come in red,blue,green,Pink,yellow etc

◆  shorter cutting intervals of down to 18",

◆  longer connection length per input of up to 165FT

◆  Premium light quality, offer CRI up to Ra90

◆  IP65 or IP68 rated and for damp or wet locations use

◆  Easy installation due to optional soAHer-free connectors

3. Product Size

4. Product Technical data

| LED Type: | SMD2835, |
| --- | --- |
| Cutting intervals: | 18” |
| Overall size: | W(08mm)*H(16mm) |
| Maximum length per input | 165FT |
| Input Voltage(V): | AC120V |
| Length /Reel: | 165FT/Reel |
| Flexible PCB Spec: | 6mm wide, |
| IP Rating: | IP65, |
| Storage temperature: | -40 o C to +80 o C |
| Operating temperature: | -20 o C to +45 o C |

- Product Spec sheet

| Item no. | Light color | Typ.color temp. | Typ.power | Typ.Lumen/3FTeter | Typ.CRI | Cutting Intervals | Maximum length per input |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AH-NF120V-0816-18-## | Super warm white | 1800K | 6w/3FT | 100lm/3FT | 80+ | 18" | 165FT |
| AH-NF120V-0816-27-## | super warm white | 2700K | 6w/3FT | 133lm/3FT | 80+ | 18" | 165FT |
| AH-NF120V-0816-30-## | warm white | 3000K | 6w/3FT | 140lm/3FT | 80+ | 18" | 165FT |
| AH-NF120V-0816-40-## | Neutral white | 4000K | 6w/3FT | 160lm/3FT | 80+ | 18" | 165FT |
| AH-NF120V-0816-50-## | Cool white | 5000K | 6w/3FT | 170lm/3FT | 80+ | 18" | 165FT |
| AH-NF120V-0816-65-## | Daylight white | 6500K | 6w/3FT | 180lm/3FT | 80+ | 18" | 165FT |
| AH-NF120V-0816-RE-## | Red |  | 6w/3FT |  |  | 18" | 165FT |
| AH-NF120V-0816-BL-## | Blue |  | 6w/3FT |  |  | 18" | 165FT |
| AH-NF120V-0816-GR-## | Green |  | 6w/3FT |  |  | 18" | 165FT |
| AH-NF120V-0816-YE-## | Yellow |  | 6w/3FT |  |  | 18" | 165FT |

- Suitable Accessories

- 6.1 Power cord connector kits

|  | IP67 2Pins In-feeder unit with 0.5M power cord |
| --- | --- |
|  | Inner Clip |
|  | Base Bracket |

- 6.2 End cap kits

|  | IP67 End cap |
| --- | --- |
|  | Inner Clip |
|  | Base Bracket |

- 6.3 Mounting Aluminium profile

|  | 165FTm , aluminium profile, width**, height** |
| --- | --- |
|  | 500mm , aluminium profile, width**, height** |
|  | 18" , aluminium profile, width**, height** |