主标题：AC LED flexible strip, SuperClassic flex series

次标题：AC120V LED Strip light,SMD5050 60LEDs/m 8W/m,450lm/m,14.1*7.3mm

- Product descriptions

- Super clear PVC extrusion make it looks very brilliant and more attractive,

- Premium quality flexible PCB layer assure good heat dissipation, electrical safety and longer maximum length per input,

- Resistant against water, UV radiation, chemicals and abrasion.

- Safe and simple connection system for easy on-site installation after cutting

- Estimated lifetime L70 at Ta25 o C :＞40000h.

2. Product features

◆  120°wide beam angle

◆  Available in homogeneous white tones,start from 2400K till 12000K

◆  a variety of light color come in red,blue,green,Pink,yellow etc

◆  longer connection length per input of up to 50m

◆  Premium light quality, offer CRI up to Ra90

◆  IP65 rated and for damp or wet locations use

3. Product Size

4. Product Technical data

| LED Type: | SMD5050 |
| --- | --- |
| LED Spec: | 22-24lm |
| LED Qty/m: | 60LEDs/m |
| Cutting intervals: | 500mm |
| Overall size: | W(14.1mm)*H(7.3mm) |
| Input Voltage(V): | AC120V |
| Dimmable(Y/N): | N/A |
| Length /Reel: | 25m/Reel,50m/Reel; |
| Standard Driver type: | Rectifier,8A, IP65 |
| FPC board Spec: | 8mm wide |
| IP Rating: | IP65 |
| Power factor: | PF>0.9 |
| Storage temperature: | -40 o C to +80 o C |
| Operating temperature: | -20 o C to +45 o C |

- Product Spec sheet

| Item no. | Input Voltage(V) | LED Qty/m | Typ.Light color | Typ.color temp. | Typ. power | Typ.Lumen/Meter | Typ.CRI | Cutting Intervals | Maximum length per input |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AH-W5050-060-27-## | AC120V | 60LEDs/m | warm white | 2700K | 8W/m | 374LM/M | 80+ | 0.5m | 50m |
| AH-W5050-060-30-## | AC120V | 60LEDs/m | warm white | 3000K | 8W/m | 392LM/M | 80+ | 0.5m | 50m |
| AH-W5050-060-40-## | AC120V | 60LEDs/m | Neutral white | 4000K | 8W/m | 450LM/M | 80+ | 0.5m | 50m |
| AH-W5050-060-65-## | AC120V | 60LEDs/m | Daylight white | 6500K | 8W/m | 480lm/m | 80+ | 0.5m | 50m |

- --The symbol ## represents the length of LED strips(Unit:m)

- --due to unavoidable LED package, electrical component and other machine tolerances, normal input power and lumen tolerance(±10%）exist;

- Suitable Accessories

Rectifier, AC120V,8A,PF≥0.9,IP65

- 7. Installation type